| STRUCTURAL - Supplement Local Technical Amendments to the 2017 6th Edition Florida Building Code |  |  |  |
| --- | --- | --- | --- |
| JURISDICTION | DOCUMENT with TECHNICAL AMENDMENT | TEXT OF TECHNICAL AMENDMENT | TAC REVEW |
| City of Winter Park | Florida Building Code, Building – Chapter 16 | Section 1609.3 Basic Ultimate wind speed. The ultimate design wind speed Vult in miles per hour, for the development of the wind loads shall be determined by Figures 1609 .3(1)A , 1609 .3(2)B and 1609.3(3)C . The ultimate design wind speed Vult for use in the design of Risk Category II buildings and structures shall be obtained from Figure 1609.3(1)A . The ultimate design wind speed Vult for use in the design of Risk Category III and IV buildings and structures shall be obtained from Figure 1609.3(2)B . The ultimate design wind speed Vult for use in the design of Risk Category I buildings and structures shall be obtained from Figure 1609.3(3)C . The ultimate design wind speeds, Vult, determined by the local jurisdiction shall be in accordance with Section 26.5.1 of ASCE 7. The exact location of wind speed lines shall be established by local ordinance using recognized physical landmarks such as major roads, canals, rivers and lake shores wherever possible. For the purpose of complying with the structural requirements related to wind loads, all buildings and structures including one and two family dwellings shall comply with the following ultimate design wind speeds Vult: 1) Risk Category I: 130 mph 2) Risk Category II: 139.9 mph 3) Risk Category III and IV: 150 mph As indicated in Figures 1609.3(1), (2) and (3) A, B, and C linear interpolation between wind contour lines is permitted. Exception: Buildings designed utilizing one of the alternate prescriptive wind design standards permitted in the Florida Building Code. This wind speed determination provided herein is an administrative amendment to the Florida Building Code for the purpose of giving guidance to designers and to provide uniformity with neighboring jurisdictions and is not a local technical amendment or change in the published Florida Building Code wind load criteria. | Structural TAC |
| Leon County Building Dept | Florida Building Code, Residential – Chapter R310-R311-R312 |  | Structural TAC and Fire TAC |