| NOTE: REVIEW COMMENTS ON CHECKLIST SENT IN BY JAMIE GASCON, MIAMI DADE 9B-72.010 Definitions. (31) Sub-category of product or construction system means a specific functionality: (a) For exterior door: 1. exterior door assemblies: roll-up, sectional, sliding, swinging, automatic; pre-engineered roof access hatches; and products introduced as a result of new technology, 2. exterior door components and products introduced as a result of new technology; (b) For windows: awning, casement, dual action, double hung, single hung, fixed, horizontal slider, projected, pass through, mullions, and products introduced as a result of new technology; (c) For panel walls: siding, soffits, exterior insulation finish system (EIFS), storefronts, curtain walls, wall louver, glass block, and products introduced as a result of new technology; (d) For roofing products: built up roofing, modified bitumen roof system, single ply roof systems, spray applied polyurethane roof system, roofing fasteners, roofing insulation, asphalt shingles, wood shingles and shakes, roofing slate, roof tile adhesives, cements-adhesives-coatings, liquid applied roof systems, underlayments, metal roofing, roofing tiles, waterproofing, roofing accessories that are an integral part of the roofing system and products introduced as a result of new technology; (e) For shutters: accordion, Bahama, storm panels, colonial, roll-up, pre-engineered equipment, protection, and products introduced as a result of new technology; (f) For skylights: skylight, and products introduced as a result of new technology; and (g) For structural components: truss plates, wood connectors, anchors, exterior coolers-freezers, insulation form systems, engineered lumber, pre-engineered air conditioner stands, structural wall components, and roof deck, and products introduced as a result of new technology. 9B-72.070 Product Evaluation and Quality Assurance for State Approval. (1) Method 1. Products specifically addressed in the code through performance criteria and standard test methods or standard comparative or rational analysis methods, which cannot be approved through the plan review and inspection process, shall demonstrate compliance with the Code through one of the following: (a) A certification mark or listing from an approved certification agency indicating that the product is in compliance with the Code. Indication of compliance shall be by certification to standards adopted by the Code. (b) A test report from an approved testing laboratory, which identifies which products are covered by the test report and provides verifiable documentation indicating the product tested complies with the Code. (c) An evaluation report from an approved product evaluation entity that covers the subject product and, based upon standard tests or standard comparative or rational analysis, or a combination thereof, indicates that the product was evaluated to be in compliance with the Code and that the product is, for the purpose intended, at least equivalent to that required by the Code. (d) An evaluation report from a Florida Registered Architect or a licensed Florida Professional Engineer developed and signed and sealed, based upon standard tests or standard comparative or rational analysis, or a combination thereof and indicates that the product was evaluated to be in compliance with the Code and that the product is, for the purpose intended, at least equivalent to that required by the Code. (e) Rational engineering analysis cannot be used in lieu of a standard test required by the Code for approval of products within the scope of the standard, except that project specific approval by the local authorities having jurisdiction in accordance with alternate methods and materials authorized in the Code. (f) A legacy report from a nationally recognized model code organization may be used if the report demonstrates compliance with the Code. (2) Method 2. Products for which there are no specific standardized tests or comparative or rational analysis methods of evaluation established as required by the Code shall demonstrate compliance with the intent of the Code through one of the following: (a) An evaluation report from an approved product evaluation entity and provides verifiable documentation indicating the product complies with the intent of the Code. (b) An evaluation report from a Florida Registered Architect or a licensed Florida Professional Engineer based on testing or comparative or rational analysis or combination thereof, which provides verifiable documentation indicating the product complies with the intent of the Code. (c) Rational engineering analysis cannot be used in lieu of a standard test required by the Code for approval of products within the scope of the standard, except that project specific approval by the local authorities having jurisdiction in accordance with alternate methods and materials authorized in the Code. (d) A legacy report from a nationally recognized model code organization may be used if the report demonstrates compliance with the code. (3) Products listed in Rule 9B-72.005, F.A.C., shall be manufactured under a quality assurance program audited by an approved quality assurance entity. (4) Evaluation Report and Test Report Documentation Requirements. All reports and documentation required in subsections 9B-72.070(1) and (2), F.A.C., shall contain the information listed below. Design drawings submitted for permitting purposes are not to be construed to be an evaluation report and do not require this information. When the Code requires a standard test as a component of a product approval using the evaluation methods, the test lab must be accredited by an approved accreditation body for the test performed. The entity issuing the evaluation report or certification is responsible to ensure that the test lab is accredited. (a) Name and address of the manufacturer, evaluation entity, engineer or architect or testing laboratory. (b) Statement of compliance with the appropriate section or standard of the Code. (c) Complete description of the product, including all drawings, manufacturer’s product designations and materials, except materials specifications identified as proprietary. (d) Technical documentation, including all substantiating data, supporting the compliance statement. Substantiating data shall include all test reports and calculations which may be referenced within the evaluation report. (e) Installation requirements. Installation instructions including attachments shall be developed by an evaluation entity, test lab or by the manufacturer’s licensed design professional. (f) Limitations and conditions of use. (g) Certification of independence in conformance with Rule 9B-72.110, F.A.C. (h) Name, title and signature of person authorized to sign on behalf of entity or signature, registration number and seal in the case of architects and engineers. (5) The following documents shall be submitted for approval of listed products or products bearing a certification mark or an approved certification agency as complying with the standards established by the Code: (a) Name and address of manufacturer and certification agency. (b) Manufacturer’s product model number or identifier and product description. (c) Indication of the standard the product was tested to. (d) Performance level of the product and conditions or limitations of use. (e) Installation requirements. (f) Agency certificate designating product certification. (6) Review and verification of installation instructions required under paragraph 9B-72.070(4)(e), F.A.C. Prior to posting installation instructions, including anchorage requirements, on the Building Codes Information System on the Internet, www.floridabuilding.org, they shall be reviewed or verified by the evaluation entity, test lab, the architect or engineer of record, or by a manufacturer’s licensed design professional. (7) Review and verification of installation instructions required under paragraph 9B-72.070(5)(e), F.A.C. Prior to posting installation instructions, including anchorage requirements, on the Building Codes Information System, www.floridabuilding.org, they shall be reviewed or verified by the certification agency or by a manufacturer’s licensed design professional. Specific Authority 553.842(1) FS. Law Implemented 553.842(6) FS. History–New 5-5-02, Amended 9-4-03, 11-22-06. 9B-72.080 Product Validation by Approved Validation Entity for State Approval. Validation of compliance with the Code shall be performed by approved validation entities through the following steps: (1) Verification that the testing, evaluation and quality assurance requirements established by Rule 9B-72.070, F.A.C., are met and that all documentation is in order. (2) Validation of the method of compliance using the validation checklist in subsection 9B-72.130(3), F.A.C. (3) Certification to the Commission that the documentation submitted for the product indicates the product complies with the Code. (4) Products listed by approved certification agencies as complying with standards established by the Code shall be approved by the Commission absent compliance with this section. Specific Authority 553.842(1) FS. Law Implemented 553.842(1) FS. History–New 5-5-02, Amended 11-22-06. 9B-72.090 Product Approval by the Commission. (1) Approval of a product or system of construction for state acceptance shall be performed by the Commission through the following steps: (a) A product manufacturer or owner of a proprietary system or method of construction, or its designee (applicant) shall apply to the Commission for approval by filing an application in accordance with subsection 9B-72.130(2), F.A.C., validated in accordance with Rule 9B-72.080, F.A.C., and submitting fees pursuant to subsection 9B-72.090(2), F.A.C. Application shall be made through the Building Codes Information System on the Internet, www.floridabuilding.org, and payment shall be by credit card or electronic check. (b) The applicant submits all documentation required and fees in accordance with Rule 9B-72.070 and subsection 9B-72.090(2), F.A.C., respectively. (c) Upon Commission acceptance of the required documentation pursuant to Rule 9B-72.070, F.A.C., and validation of compliance with the Code pursuant to Rule 9B-72.080, F.A.C., the Commission may approve the product for use statewide in accordance with its approval and limitations of use unless credible evidence is provided questioning the validity of the documentation submitted in support of the application for approval. (d) Approval shall be valid until such time as the product changes decreasing the product’s performance the standards or provisions of the Code affecting the product change, or the approval is otherwise suspended, revoked, or superseded by a Commission approved revision to the approval. Changes to the Code shall not be construed as voiding the approval of products previously installed in existing buildings provided such products met building code requirements at the time the product was installed. (e) When a new edition of the Code does not require a material or substantive change for an approved product, the manufacturer of the approved product shall affirm that his or her approved product meets the new edition of the Code. Self-affirmation is subject to review and verification by the Program Administrator. (f) Manufacturer or the manufacturer’s designee shall notify the Commission if it makes changes to the product which decrease the product’s performance. (g) Manufacturer shall notify the Commission when the quality assurance requirements of subsection 9B-72.070(3), F.A.C., are no longer in place. (2) Fees for state approval of products. (a) Fee for approval, Three Hundred Dollars ($300.00) per sub category or product. The Commission shall review annually and adjust fees accordingly. (b) Fee for reinstatement after suspension, Fifty Dollars ($50.00) per product, plus billable staff hours at Fifty Dollars ($50.00) per hour, plus consultant fees. (c) Fees for approval of evaluation entities, certification agencies, testing laboratories and validation entities; for first time approval, Five Hundred Dollars ($500.00), annual renewal fee, One Hundred Dollars ($100.00), revision fee, One Hundred Dollars ($100.00). (d) Fee for revision of an existing approval, Three Hundred Dollars ($300.00) for a revision that results in a material change to the performance of a product or product design specification or both, and which may include addition of products within the same subcategory. (e) Fee for editorial revisions of an existing product approval that does not result in material change to the performance of a product or product design specification or both, One Hundred Dollars ($100.00). (f) Fee for affirmation of an existing product approval for compliance with a new edition of the standards adopted by the Code, Fifty Dollars ($50.00). (3) Applications, affirmations and revisions shall be made through the Building Codes Information System on the Internet, www.floridabuilding.org, and payment shall be by credit card or electronic check. Specific Authority 553.77(1)(i), 553.842(1) FS. Law Implemented 553.842(1) FS. History–New 5-5-02, Amended 9-4-03, 11-22-06. 9B-72.100 Approval of Product Evaluation Entities, Product Validation Entities, Testing Laboratories, Certification Agencies, Quality Assurance Agencies and Accreditation Bodies. (1) SEE RULE (2) Approved Validation Entities. (a) An entity shall be approved by the Commission as a validation entity if it is a Commission approved evaluation entity, testing laboratory or certification agency and it certifies to the Commission compliance with standards established by the Code or intent of the Code. Architects and engineers licensed in this state are also approved to conduct validation for the state approval. Validation by an approved testing laboratory acting as a validation entity shall be limited to the scope for which they are accredited. (b) An entity may be approved as a validation entity after applying to the Commission for approval. Applications must be submitted in accordance with subsections 9B-72.130(1) and 9B-72.090(3), F.A.C., including a Certificate of Independence in accordance with Rule 9B-72.110, F.A.C., and fees submitted pursuant to subsection 9B-72.090(2), F.A.C. Application shall be made through the Building Codes Information System on the Internet, www.floridabuilding.org, and payment shall be by credit card or electronic check. (c) Approvals shall be valid until such time as Commission approval requirements change, the entity no longer qualifies under current requirements or the approval is suspended or revoked. (3) Approved Testing Laboratory Criteria. Approval by the Commission is limited to the scope of accreditation established by approve accreditation entities. (a) An entity shall be approved by the Commission as an approved testing laboratory if it complies with one of the following. Approval shall be limited to those procedures listed on the certificate of accreditation or accreditation listing issued by the accreditation body. 1. Testing laboratories accredited by American Association for Laboratory Accreditation (A2LA) that meet the requirements of the International Organization for Standardization/International Electrotechnical Commission (ISO/IEC) Guide 17025: General Requirements for the Competence of Calibration and Testing Laboratories or other standard certified as equivalent by the accrediting entity pursuant to Rule 9B-72.180, F.A.C., and approved by the Commission. [NO CHANGES TO TEXT] 9B-72.130 Forms. The following forms are adopted for use in reference to the Product Evaluation and Approval System. Copies of these forms are available from the Department of Community Affairs, Codes and Standards Section, 2555 Shumard Oak Boulevard, Tallahassee, Florida 32399, and via the Building Codes Information System on the Internet, www.floridabuilding.org. (1) Florida Building Commission, Application for Organization/Entity Approval, Form No. 9B-72.130(1), updated October 11, 2005 (electronic version). (2) Florida Building Commission, Application for State Product Approvals, Form No. 9B-72.130(2), updated October 11, 2005 (electronic version). (3) Validation Checklist for State Approval, Form No. 9B-72.130(3), updated September 4, 2003 (electronic version). Specific Authority 553.842(1) FS. Law Implemented 553.842(1) FS. History–New 5-5-02, Amended 9-4-03, 11-22-06. 9B-72.160 Revocation or Modification of Product Approvals and Entity Certifications. (1) Product Approval Revocation or Suspension. (a) Any product approval shall be revoked or suspended for any of the following reasons: 1. Failure to maintain certification, evaluation reports or testing in good standing with a Commission approved entity which conducted the testing or comparative or rational analysis, or combination thereof on which the product approval is based. 2. Suspension or revocation of the certification, evaluation report or testing report issued by a Commission approved entity on which the approval is based, for just cause. 3. Failure to maintain quality assurance programs for the manufacture of the approved products as required by this document. 4. Failure to correct manufacturing deficiencies required to bring the product within specifications of the originally approved product or alternatively to demonstrate in a manner consistent with this document, that the product’s performance complies with the standards established by the Code. 5. Advertising and sales of the product for uses not consistent with conditions or limitations of its approval. 6. Determination that the product was approved based on misrepresentations in the application for approval. 7. Failure of the manufacturer to cooperate with a Commission ordered investigation. (b) The Commission may suspend the approval of a product based on any provision of subsection 9B-72.160(1), F.A.C., until such time as the manufacturer demonstrates the product is currently in compliance with this document. (c) The Commission shall initiate an investigation based on a written complaint containing substantial material evidence by any substantially affected party. (d) The Commission shall clearly post the status of product approvals, denials, or suspensions on its website, the Florida Building Codes Information System, www.floridabuilding.org. (2) Revocation or suspension of evaluation entity, certification agency, testing laboratory, validation entity, quality assurance agencies or accreditation body approval. (a) The Commission shall revoke or suspend the approval of any evaluation entity, certification agency, testing laboratory, quality assurance agency, or validation entity other than those specifically identified in Section 553.842(9)(a), F.S., for one or more of the following reasons: 1. Failure to maintain accreditation by a Commission approved accreditation body. 2. Suspension or revocation of accreditation by a Commission approved accreditation body for failure to meet Commission accreditation standards or equivalent pursuant to Rules 9B-72.100 and 9B-72.180, F.A.C. 3. Determination by the Commission that any requirement set forward in this document has been violated. 4. Determination that the criteria for independence from any manufacturer set forth in Rule 9B-72.110, F.A.C., has been violated. 5. Determination that the entity is not independent pursuant to Rule 9B-72.110, F.A.C., of any competing manufacturer of the manufacturer to whom the entity provided services on which Florida jurisdictions’ product approval is based. 6. An entity has misrepresented its accreditations or other material information on its application for approval. 7. Failure to conduct investigations of products authorized by Rule 9B-72.170, F.A.C. (b) The Commission may revoke or suspend the approval of any approved accreditation body for failure to maintain accreditation programs which comply with subsection 9B-72.100(6), F.A.C., or any material misrepresentation of its independence or substantive information on its capabilities or policies and procedures and failure to cooperate in investigations of those it accredits. (c) Commission suspensions under subsection 9B-72.160(2), F.A.C., shall remain in effect until such time as the entity demonstrates to the Commission that it is in compliance with said requirement. (d) The Commission shall initiate an investigation based on a written complaint providing substantial material evidence provided by any substantially affected party. (e) The Commission shall clearly post the status of approved evaluation entity, certification agency, testing laboratory, validation entity, quality assurance agency and accreditation body approval, suspension or revocation on its website list of approved entities. (3) Incomplete Product Approval or Entity applications. Any application that has no activity and is not complete within 180 days from the date of initial filing shall be denied. | Only Allow Certifications for products that have been tested to standards referenced in the code, do not allow certifications of products that have no test standard(s). (SEE PAVWG Page #1) Establish a rule that if the test standard requires rational analysis, the method for demonstrating compliance must be by an evaluation report. (SEE PAVWG PAGE #3) Window Workgroup Recommendations Type, and grade of anchor, and/or manufacturers anchor specifications, including minimum nominal size, length, minimum penetration into substrate and minimum edge distances. Type, physical dimensions, material and grade of any insert accessory item or strap used.if applicable. Spacing of anchors, inserts shims, accessory items and straps. at all points of attachment. Illustrated diagrams of the attachment of the product to the structure. Evaluation must be signed and sealed. (SEE PAVWG PAGE #2) The VALIDATOR must perform a TECHNICAL REVIEW sufficient to determine that the evaluator has complied with acceptable standards of engineering principals”. And, “the VALIDATOR must provide an engineering verification that the evaluation complies with the code.” Additionally, “a hardcopy of the application, complying with all aspects of the RULE (Florida Board of Professional Engineers adopted Rule 61G15-36, FAC), must be filed with the Florida Building Commission to supplement the online filing”. This applies to Registered Architects as well. (SEE PAVWG PAGE#2) Architects and Engineers validating shall sign and seal hardcopy of validation checklist. (SEE PAVWG PAGE #2) If validation is defined as a technical review, then the evaluation engineer does not need to be an independent third-party from the manufacturer. Manufacturer’s engineer can do the evaluation, provided the engineer is a Florida PE or RA who has taken the core building code course. (SEE PAVWG PAGE #2) Validation for the test report method should be a technical review to verify that the reported date demonstrates compliance with the Code, and the correct test was done. (See PAVWG Page #3) Validator shall retain all documentation on application (test reports, calculations, samples, etc.) for the time required by Florida law for documents retention of public documents, or for as long as a product is approved based on their validation, whichever is longer. (See PAVWG Page #5) Time limits for State approval of products shall be established. (SEE PAVWG PAGE #5) Testing laboratories shall not validate their own test report application (SEE PAVWG PAGE #3) Test labs shall not be allowed to conduct validation (SEE PAVWG PAGE #3) A mandatory training program for validator(s), and require validator(s) to be accountable for their mistakes when not complying with the requirements in Rule 9B-72, shall be established. Providing a system of accountability regarding the validator (i.e., certification or a contract with the validator). (SEE PAVWG PAGE #5) Products approved by certification method should have the online form reviewed to insure that the information presented is supported by the certification documents. The review can be done by the certification agency or any approved validation entity (operating within their scope of accreditation, if a test lab). The technical accuracy of the certification is the responsibility of the certification agency. (SEE PAVWG PAGE #1) Validation Checklists (a)Certification method (b)Test report (c)Evaluation Entity (d)Evaluation report from an Architect or Engineer (SEE PAVWG PAGE #1,2,3,4) Penalties for validators that incorrectly validate applications shall be developed. (SEE PAVWG PAGE #5) POC shall be proactive and discuss the referral to licensing boards and accreditation entities complaints on improper evaluations and validations. (SEE PAVWG PAGE #5) | POC Recommendation – Approved with amendments BCIS – - Revise to allow multiple certification agencies per application provided there is a third party validation of the application (SEE PAVWG PAGE #1) Defer this function until such time as the system is due for total system revamp. 2 . Revise to allow the uploading of an evaluation report for the same product as part of the certification compliance method. 3. For evaluation reports issued by an approved evaluation entity, other than architects or engineers, the certificate of independence is not required for each application. 4. Checklist – Criteria for technical validation - Certification agency compliance method has to be validated - Checklist per compliance method - Acknowledgement of hardcopies. FUNDING For BCIS A. Require future PA administrator to reimburse DCA the admin charge per application ($25) as part of future agreement. ---Discussed and contractor/admin will provide for the $25 out of the current fees. (Not an issue) 9B-72.010 (31) (e) For shutters: accordion, Bahama, storm panels, fabric storm panel, colonial, roll-up, pre-engineered equipment, protection, and products introduced as a result of new technology; POC Recommendation: (Already covered by Rule 9B-72.070) No Action Needed POC Recommendation: Revise Rule 9B-72.070(1) to add a new item (g) to read as follows – Products required to demonstrate compliance using referenced testing standard(s) and also requiring rational analysis that falls outside the scope of the agency certificate designating product certification, must demonstrate compliance using an evaluation report from an approved evaluation entity. (e) Installation requirements. Installation instructions including attachments shall be developed by an evaluation entity, test lab or by the manufacturer’s licensed design professional Exception: Installation instructions for windows including attachment at minimum shall include the following: 1. Type, and grade of anchor, and/or manufacturers anchor specifications, including minimum nominal size, minimum penetration into substrate and minimum edge distances. 2. Type, physical dimensions, material and grade of any accessory item or strap, if applicable. 3. Spacing of anchors, shims, accessory items and straps. 4. Illustrated diagrams of the attachment of the product to the structure. POC Recommendation: Covered, no action is needed. (e) Installation requirements. Exception: Installation instructions for windows including attachment at minimum shall include the following: 1. Type, and grade of anchor, and/or manufacturers anchor specifications, including minimum nominal size, minimum penetration into substrate and minimum edge distances. 2. Type, physical dimensions, material and grade of any accessory item or strap, if applicable. 3. Spacing of anchors, shims, accessory items and straps. 4. Illustrated diagrams of the attachment of the product to the structure. POC Recommendation: Revise Rule 9B-72.080 as follows: 9B-72.080 Product Validation by Approved Validation Entity for State Approval. - Administrative Validation: Validation of comliance with the Code using a certification mark or listing from an approved certification agency, an evaluation report from an approved product evaluation entity, and an evaluation report from a Florida Registered Architect or licensed Florida Professional Engineer shall be performed by an approved validation entity through the following steps: (1a) Verification that the certification, testing, evaluation and quality assurance requirements established by Rule 9B-72.070, F.A.C., are met and that all documentation is in order. (2b) Validation of the method of compliance using the validation checklist in subsection 9B-72.130(3), F.A.C. Signed and sealed hard copy of the Validation Checklist must be provided to the Program System Administrator when the validation entity is a Florida registered architect or licensed Professional Engineer. (3c) Certification to the Commission that the documentation submitted for the product indicates the product complies with the Code. (4d) Products listed by approved certification agencies as complying with standards, that do not include rational analysis, established by the Code shall be approved by the Commission upon validation using Administrative Validation. Code shall be approved by the Commission absent compliance with this section. (2). – Technical Validation (a) Technical Validation of an Evaluation Report: Validation of compliance with the Code using an evaluation report from an approved evaluation entity that is not an independent third-party from the manufacturer shall be performed by an approved validation entity using the validation steps of the Administrative Validation and the following steps: (1) Determination that the evaluator has complied with acceptable standards of engineering principles. (2) Engineering verification that the evaluation complies with the Code. (3) Copy of the application complying with all aspects of rule 61G15-36 F.A.C., must be filed with the Commission. (b) Technical Validation of a Test Report: Validation of compliance with the Code using a test report from an approved test lab shall be performed by an approved validation entity using the validation steps of the Administrative Validation and the following steps: - Verify that the reported data demonstrates compliance with the Florida Building Code, and - Verify that the correct test was performed. (c) Technical Validation of a Installation Instructions including attachments: Installation instructions including attachments developed by an entity that is not an independent third-party from the manufacturer shall be performed by an approved validation entity using the validation steps 1 through 3 of item 2(a) above. Exception: Technical validation is not required if the installation intruction including attachments are verified by the product certification agency or the product evaluation entity. (3). Approved Validation entities must retain all documentation of the product applications (test reports, calculations, samples, etc.) as required by Florida statutes or as long as the products are approved, based on their validation, whichever is longer. POC RECOMMENDATION NO ACTION NEEDED Already defined POC Recommendation: Revise Rule 9B-72 100(2)(a) to read as follows: ACTION #1 (a) An entity shall be approved by the Commission as a validation entity if it is a Commission approved evaluation entity, testing laboratory or certification agency and it certifies to the Commission compliance with standards established by the Code or intent of the Code. Architects and engineers licensed in this state are also approved to conduct validation for the state approval. Validation by an approved testing laboratory acting as a validation entity shall be limited to the scope for which they are accredited. An approved testing laboratory acting as a validation entity shall not validate its own test report application when using the test report compliance method. ACTION # 2 (a) An entity shall be approved by the Commission as a validation entity if it is a Commission approved evaluation entity, testing laboratory or certification agency and it certifies to the Commission compliance with standards established by the Code or intent of the Code. Architects and engineers licensed in this state are also approved to conduct validation for the state approval. Validation by an approved testing laboratory acting as a validation entity shall be limited to the scope for which they are accredited. Testing Laboratories are not allowed to conduct validations. POC Recommendation: Action 2 as amended. Staff Recommendation: Add section: 9B-72.100 (2)(d) Continuing Education Requirements for Validation Entities: Entities who are approved as Validation Entities by the Commission must, as a condition for each renewal, provide proof of completion of at least 4 hours classroom or interactive distance learning hours of continuing education in one or more courses on the Florida Building Code, or attend 2 (two) Product Approval Program Oversight Committee Meetings (POC), or special Product Approval training conducted by the agency or its administrator. The Committee discussed requiring Continuing Education for Validation Entities, however there were a number of logistic and implementation issues that prevented the Committee from making a determination at this meeting. It was determined that further discussion on this subject is necessary at the May meeting. Note: Because of the current improvements to the validation process under the proposed changes to rule 9B-72 the POC decided that continuing education for the validators is not necessary and the task requiring continuing education should be removed from the commission “workplan”. POC Recommendation: See attached Staff Recommendation: Already covered in the rule (NO ACTION NEEDED) Staff Recommendation: No Rule language needed Staff Recommendation: Need Discussion |
| --- | --- | --- |