Energy Simulation Tool Approval Technical Assistance Manual

Developed and Produced by JM Jadu Corp	2010 Florida Building Code, Energy Conservation

West Palm Beach, FL				Document Number: TAM-2010-1.0

This Page is intentionally blank

Contents

Introduction	5

1	Overview of Process	6

1.1	Special Terms and Definitions	6

1.2	Approval Requirements	7

1.3	Optional capabilities	9

2	Types of Approval	9

2.1	Full Approval & Re-Approval of Compliance Software Programs	10

2.2	Approval of New Features & Updates	11

2.3	Challenging Compliance Software Program Approval	12

3	Vendor Requirements	12

3.1	Vendor Certification Statement	12

3.2	Availability to Commission	12

3.3	User Support	13

3.4	Compliance Software Vendor Demonstration	13

3.5	Application Checklist	13

3.6	Where to Send Application	14

4	User’s Manual and Help System Requirements	15

4.1	Statement	17

5	Alternative Compliance Software Program Tests	17

6	Residential Energy Compliance Software Programs	18

6.1	General Requirements	18

6.2	Minimum Capabilities	19

6.3	Compliance Report	20

6.4     Residential Energy Performance Testing	22

6.4.1          Residential Accuracy Test cases	22

6.4.2	 Reference Test Cases	23

6.2.3	Testing Procedures	25

7	Commercial Energy Compliance Software Programs	26

7.1	General Requirements	27

7.1.1	Trade-Offs Limited to Building Permit	28

7.2	Performance-based Compliance	29

7.2.1	Calculation Procedure	29

7.2.2	Calculation Methods	30

7.2.3	Error Handling	30

7.2.4	Climate Data	30

7.2.5	Utility Rates	31

7.3	Managing User Input	31

7.3.1	Building Descriptor Inputs and Restrictions	31

7.3.2	User Interface	31

7.3.3	Compulsory Input Checks	32

7.3.4	Handling of Missing Inputs	32

7.3.5	Validity Checks	33

7.3.6	Handling Invalid Input	33

7.3.7	Consistency Checks	33

7.3.8	Handling Inconsistent Input	33

7.4	Documentation	34

7.4.1	Compliance Report	34

7.4.2	Additional documentation (Section 506.4.2)	35

7.5	Commercial Tests Verification

7.5.1		Error! Bookmark not defined.Tests to verify thermal loads:  ASHRAE Standard 140-2007

7.5.2	Tests to verify proosed design building is modeled correctly	Error! Bookmark not defined.

7.5.3	Tests to verify that the Standard Reference Design building is created correctly	Error! Bookmark not defined.

7.5.4	Reference building auto-generation test 	Error! Bookmark not defined.

7.5.5	Test result format	Error! Bookmark not defined.

8       References	Error! Bookmark not defined.

Appendix A. Residential Form 405- Preferred Content and Format	41

Appendix B. Commercial and High-Rise Residential Form 506 – Preferred Content and Format	Error! Bookmark not defined.

# Introduction [new, reformatted from 1.0]

As part of the new 2010 Florida Building Code, Energy Conservation, the Florida Building Commission will be charged with the responsibility of approving energy simulation tools.

This Manual explains the requirements for approval of energy simulation tools (also referred to as compliance software programs) used to demonstrate compliance with the 2010 Florida Building Code, Energy Conservation--the “energy code” for residential and nonresidential building designs. An “energy simulation tool” is defined by the code as an approved software program or calculation-based methodology that projects the annual energy use of a building. Compliance software programs are used to demonstrate code compliance with the energy code by the performance approach for building design. The code requirement for residential compliance by the Simulated Performance Alternative is specified in Section 405 and Normative Appendix B of the energy code. The commercial (and residential high-rise) code requirements for compliance by Total Building Performance are specified in Section 506 and Normative Appendix B of the energy code. Compliance software programs are used in the performance approach to demonstrate compliance with the energy code for building designs. The Florida Building Commission, “the Commission”, develops and implements the energy code.

# 1	Overview of Process

The purpose of this Technical Assistance Manual (“this Manual”) is to outline the Florida Building Commission’s approval process for compliance software programs and to define the procedures, minimum requirements and assumptions against which compliance software programs should be evaluated. The performance compliance requirements and procedures apply to both residential and nonresidential buildings. The procedures and processes described in this Manual are advisory in nature; an alternative compliance procedure to that described in this document is acceptable as long as such alternative is approved by the Commission and designed to preserve the integrity of the performance code compliance process.

The reference procedures and method described in this manual establish the basis of comparison for all code compliance software. The approval process as outlined in this manual ensures that a minimum level of energy conservation is achieved regardless of the compliance software used. This is accomplished by:

- having candidate compliance software pass a series of industry standard tests,

- identifying minimum input which may be used to generate the standard reference design,

- defining standard reports output requirements, and

- compliance software vendor-certification to the requirements in this Manual and the energy code.

## 1.1	Special Terms and Definitions

There are a few other special terms that are used in this Manual. The Commission approves the use of energy simulation tools (compliance software programs) for code compliance. Commission approval means that the Commission accepts the applicant's certification that a compliance software program meets the requirements of the energy code and this Manual.  [Definitions are reformatted for clarity]

- "Compliance" means that a building design in an application for a building permit complies with the energy code and meets the requirements described for building design standards.

- “Compliance supplement” is an independent user's manual for the compliance software program.

- "Energy code" means the 2010 Florida Building Code, Energy Conservation.

- “Energy simulation tool” is defined by the energy code to mean an approved software program or calculation-based methodology that projects the annual energy use of a building.

- “Proposed Design” means a description or computer representation of the proposed building used to estimate annual energy use for determining compliance based on total building performance or design energy cost.

- “Standard Reference Design” means a version of the Proposed Design that meets the minimum requirements of the energy code and is used to determine the maximum annual energy use requirement for compliance based on total building performance.

- “Vendor” is the proponent of a candidate compliance software program.

## 1.2	Approval Requirements [was 1.4]

For the vendor, the process of receiving approval of a compliance software program includes preparing an application, working with the Commission staff to answer questions from either Commission staff or the public, and providing any necessary additional information regarding the application. The application includes the four basic elements outlined below. The Commission staff evaluates the compliance software program based on the completeness of the application and its overall responsiveness to staff and public comment.

The basic requirements for approval include:

- Minimum energy code compliance capabilities:

Compliance software programs shall have all the required capabilities identified in the 2010 Florida Building Code, Energy Conservation. The requirement for residential compliance is specified in Section 405 and Normative Appendix B of the Energy Code.   The nonresidential requirements are specified in Section 506 and Normative Appendix B of the Energy Code.

- Accuracy of energy simulation tool:

The compliance software program shall demonstrate acceptable levels of accuracy by performing and passing the required certification tests discussed in Chapters 6 (residential) and 7 (commercial) of this Manual as modified by the Vendor to address Florida’s specific climate conditions.

The compliance software program vendor performs the certification tests in Chapters 6 or 7, respectively, for residential or commercial projects. The vendor conducts the specified tests, evaluates the results and certifies in writing that the compliance software program passes the tests. The Commission may perform spot checks and may require additional tests to verify that the proposed compliance software program is appropriate for code compliance purposes.

When energy analysis techniques are compared, two potential sources of discrepancies include 1) the differences in user interpretation when entering the building specifications, and 2) the differences in the compliance software program's algorithms (mathematical models) for estimating energy use. The approval tests minimize differences in interpretation by providing explicit detailed descriptions of the test buildings that must be analyzed.  For differences in the compliance software’s algorithms, the Commission allows algorithms that yield equivalent results. [Deleted per JN comment & 10/17 work group recommendation]

- User’s Manual or help system:

The vendor shall develop a user’s manual and/or help system that meets the specifications in Chapter 4 of this manual.

- Program support and reporting forms:

The vendor shall provide ongoing user and enforcement agency support as described in Chapter 3 of this manual.

The Commission may hold one or more workshops…of this manual and the Energy Code.  [Text moved to new section 3, Vendor reqts.]

In addition to explicit and technical criteria, Commission approval may also depend upon the Commission's evaluation of:

- Enforceability in terms of reasonably simple, reliable, and rapid methods of verifying compliance and,

- application of energy conservation features modeled by the compliance software and,

- the inputs used to characterize those features by the compliance software users and,

- dependability of the installation and energy savings of features modeled by the compliance software program.

## 1.3	Optional capabilities [was 1.5]

Optional capabilities are a special class of capabilities and user inputs that are not required of all compliance software but may be included at the option of the vendor. Additional optional capabilities may be proposed by vendors. For both cases, the Commission reserves the right to disapprove the certification application for a specific optional capability if there is not compelling evidence presented in the public process showing that the optional capability is sufficiently accurate and suitable to be used for compliance with the energy code. In addition, energy conservation measures modeled by optional capabilities shall be capable of being verified by local enforcement agencies.

The Commission's purpose in approving additional optional capabilities is to accommodate new technologies which have only begun to penetrate the market and new modeling algorithms. Optional capabilities which evaluate measures already in relatively common use shall have their standard design for the measure based on the common construction practice for that measure since common practice is the inherent basis of the energy code for all measures not explicitly regulated. For example, the Commission has no interest in an optional capability that evaluates the energy impacts of dirt on windows unless a new technology produces substantial changes in this aspect of a building relative to buildings without this technology. The burden of proof that an optional capability should be approved lies with the vendor.

# 2	Types of Approval [was 1.6]

This Manual addresses two types of compliance software program approval: full program approval (including amendments to programs that require approval), and approval of new program features and updates.

If compliance software program vendors make a change to their programs as described below, the Commission shall again approve the program.

- Any compliance software program change that affects the energy use calculations for compliance,

- the modeling capabilities for compliance,

- the format and/or content of compliance forms, or

- any other change which would affect a building's compliance with the energy code requires another approval.

Changes that do not affect compliance with the energy code such as program changes to the user interface may follow a simplified or streamlined procedure for approval of the changes. To comply with this simpler process, the compliance software program vendor shall certify to the Commission that the new program features do not affect the results of any calculations performed by the program, shall notify the Commission of all changes and shall provide the Commission with one updated copy of the program and user's manual. Examples of such changes include fixing logical errors in computer program code that do not affect the numerical results (bug fixes) and new interfaces.

## 2.1	Full Approval & Re-Approval of Compliance Software Programs

The Commission requires program approval when a candidate compliance software program has never been previously approved by the Commission, when the compliance software vendor makes changes to the program algorithms, or when any other change occurs that in any way affects the compliance results. The Commission may also require that all currently approved compliance software programs be approved again whenever substantial revisions are made to the energy code or to the Commission's approval process. The Commission may change the approval process and require that all compliance software programs be approved again for several reasons including:

- If the energy code undergoes a major revision that alters the basic compliance process, then compliance software would have to be updated and re-approved for the new process.

- If new analytic capabilities come into widespread use, then the Commission may declare them to be required compliance software capabilities, and may require all compliance software vendors to update their programs and submit them for re-approval.

When re-approval is necessary, the Commission will notify all compliance software vendors of the timetable for renewal.

A compliance software program must be re-approved for new optional modeling capabilities when the vendor adds those optional capabilities. The vendor shall provide a list of the new optional capabilities and demonstrate that those capabilities are documented in revised user documentation. This may not include computer runs previously submitted.

Re-approval shall be accompanied by a cover letter explaining the type of amendment(s) requested and copies of other documents as necessary. The timetable for re-approval of amendments is the same as for full program approval.

## 2.2	Approval of New Features & Updates

Certain types of changes may be made to previously approved compliance software through a streamlined procedure, including implementing a computer program on a new machine and changing executable program code that does not affect the results.

Modifications to previously approved compliance software including new features and program updates are subject to the following procedure:

•	The compliance software program vendor shall prepare an addendum to the compliance supplement or compliance software program user's manual, when new features or updates affect the outcome or energy conservation measure choices, describing the change to the compliance software. If the change is a new modeling capability, the addendum shall include instructions for using the new modeling capability for compliance.

•	The compliance software program vendor shall notify the Commission by letter of the change that has been made to the compliance software program. The letter shall describe in detail the nature of the change and why it is being made. The notification letter shall be included in the revised Compliance Supplement or compliance software user’s manual.

•	The compliance software program vendor shall provide the Commission with an updated copy of the compliance software program and include any new forms created by the compliance software (or modifications in the energy code reports).

•	The Commission may approve the change, request additional information, refuse to approve the change or require that the compliance software vendor make specific changes to either the Compliance Supplement addendum or the compliance software program itself.

With Commission approval, the vendor may issue new copies of the compliance software with the Compliance Supplement addendum and notify compliance software program users and building officials.

[2.1 Alternative Compliance Software Program Tests, moved to 5]

## 2.3	Challenging Compliance Software Program Approval [was 2.2]

Any challenge to software approval by the Florida Building Commission shall be in accordance with Chapter 120, Florida Statutes.

# 3	Vendor Requirements

Commission approval of compliance software programs is intended to provide flexibility in complying with the energy code. However, in achieving this flexibility, the compliance software program shall not degrade the energy code or evade the intent of the energy code to achieve a particular level of energy conservation.  [Text moved here from section 1.4, 4)]

The vendor has the burden of proof to demonstrate the accuracy and reliability of the compliance software relative to the test methods and to demonstrate the conformance of the compliance software to the requirements of this manual and the energy code. [Text moved here from section 1.4,4)]

Each compliance software vendor shall meet all of the following requirements as part of the compliance software approval process and as part of an ongoing commitment to users of their particular program. [Text moved here from section 1.4, 4)]

## 3.1	Vendor Certification Statement [was 1.3]

The vendor shall follow the procedure described in this document to certify to the Commission that the compliance software meets the requirements of the energy code and the criteria in this document for:

- Accuracy and reliability when compared to the standard tests; and

- Ability to generate minimum energy code required standard reference design from user inputs; and

- Suitability in terms of the accurate calculation of the correct energy budget, the printing of standardized forms, and

- The documentation on how the program demonstrates compliance.

## 3.2	Availability to Commission [was 3.1.1]

All compliance software program vendors are required to submit at least one fully working program version of the compliance software to the Commission’s staff, and shall provide the Commission’s Energy Technical Advisory Committee and interest groups access to the software for review during the approval process.

## 3.3	User Support [was 3.1.2]

Compliance software vendors shall offer support to their users with regard to the use of the compliance software for compliance purposes.

## 3.4	Compliance Software Vendor Demonstration

The Commission may request compliance software vendors to physically demonstrate their program's capabilities. One or more demonstrations may be requested before approval is granted. [was 3.1.3] The Commission may hold one or more workshops with public review and vendor participation to allow for public review of the vendor's application. Such workshops may identify problems or discrepancies that may necessitate revisions to the application. [Text moved here from section 1.4,4)]

## 3.5	Application Checklist [Moved here from second 1.5]

The following items shall be included in an application package submitted to the Commission for compliance software approval:

- Compliance Software Vendor Certification Letter

The Vendor shall submit a signed compliance software letter, certifying that the compliance software meets the energy code requirements, including accuracy and reliability when used to demonstrate compliance with the energy code.

- Computer Runs

The required formats for building information reports are electronic Portable Document File (PDF). Reports will be automatically generated by the software. Each page of the report will have a header with the software name and date.

- Compliance Supplement and User's Manual

The vendor shall submit a complete copy of their compliance software program’s user's manual, including material on the use of the compliance software for compliance purposes and an executable machine readable copy of the compliance software program for random verification of compliance analyses. The vendor shall provide weather data for major Florida cities having weather stations all climate zones.

[Work group recommended changing “machine” to “pdf”.

## 3.6	Where to Send Application [Moved here from second 1.5]

Two copies of the full application package should be sent to:

Florida Building Commission

Building Codes and Standards Office

2555 Shumard Oak Blvd.

Tallahassee, Florida 32399-2100

Following submittal of the application package, the Commission may request additional information. This additional information is often necessary due to complexity of compliance software. Failure to provide such information in a timely manner may be considered cause for rejection or disapproval of the application. A resubmittal of a rejected or disapproved application will be considered a new application.

# 4	User’s Manual and Help System Requirements

The compliance software program user’s manual and help system should shall be written in a clear and concise manner.

Each compliance software program vendor is required to publish a compliance supplement or an independent user's manual which explains how to use the compliance software for compliance with the energy code. The manual may also exist in electronic form, either on the user’s workstation or web enabled. The document should shall deal with compliance procedures and user inputs to the compliance software. Both the compliance software program user’s manual and help system should shall positively contribute to the user's ability and desire to comply with the energy code and to the enforcement agency's ease of verifying compliance. The compliance software program user’s manual and help system should minimize or reduce confusion and clarify compliance applications. The Commission may reject a compliance software program certification submittal whose compliance software program user’s manual and help system does not serve or meet these objectives.

The compliance software program user’s manual and help system should shall:

•	Describe the specific procedures for using the compliance software for compliance with the energy code.

•	Provide instructions for preparing the building input, using the correct inputs, and using each of the approved optional capabilities (or exceptional methods) for which the compliance software is approved.

•	Explain how to generate the energy code compliance reports and related compliance documentation. A sample of properly prepared compliance documentation shall be included as part of the manual or help system.

The compliance software program user’s manual and help system serve two major purposes:

•	It helps building permit applicants and others use the compliance software program correctly, and guide them in preparing complete compliance documentation to accompany building permit applications.

•	It helps enforcement agency staff plan check permit applications for compliance with the energy code.

The compliance software program user’s manual and help system serves as a crucial performance method reference in resolving questions concerning specific compliance software program attributes, approved modeling capabilities and procedures in the context of both compliance and enforcement.

The compliance software program user’s manual and help system should shall contain a chapter or section on how to model buildings for compliance and how to prepare a building input file for a compliance run. The following are examples of topics to include:

•	What surfaces to model (exterior, interior floors, etc.);

•	How to enter data about these surfaces;

•	How to model exterior shading (fins, overhangs, etc.);

•	Appropriate zoning for compliance modeling;

•	Selection of correct occupancy types;

•	How to model similar systems;

•	How to model buildings or portions of a building with no heating or cooling;

•	Requirements for written justification and additional documentation on the plans and in the specifications for exceptional items;

•	Program modeling limitations; and

•	This Manual as required reading.

All program capabilities should be described in sufficient detail to eliminate possible confusion as to their appropriate use. While references to the compliance software program's regular user’s manual are acceptable, a complete listing of all inputs and/or commands necessary for compliance should be included in the compliance software program user’s manual and help system.

The compliance software program user’s manual and help system should shall contain clear and detailed information on how to use the compliance software to model buildings for compliance with the energy code.

The compliance software program user’s manual and help system should include the following:

- Description of the value or values associated with each of input.

- Restrictions on each variable.

- Listing of the range beyond which inputs are unreasonable for any variable.

- Description of options for any user-defined variable.

- A chapter or section which covers each energy code output report.

Appendices, as needed, to provide any additional background information that is not crucial in explaining the basic functioning of the program for compliance. For example:

- An appendix may contain variations of compliance forms as described above.

- An appendix may include a series of construction assembly forms to aid the compliance software program user.

- An appendix may reprint important sections of the manual that are crucial to modeling buildings correctly for compliance with the compliance software program.

- Although the organizational format is not fixed, all information contained in the compliance software program user’s manual and help system should shall be easy to find through use of a table of contents, an index, or through a context sensitive help system.

## 4.1	Statement

The following statement shall appear, in a box, within the first several pages of the compliance software program user’s manual and help system

[Insert Name of Calculation Method] may be used to show compliance with 2010 Florida Building Code, Energy Conservation only when the following reference documents are readily available to the program user:

- 2010 Florida Building Code, Energy Conservation

- Energy Simulation Approval: Technical Assistance this Manual (TAM 2010-1.0)

## 5 4.2	Alternative Compliance Software Program Tests [was originally 2.1; duplicate deleted from 7.1]

Sections Chapters 6.4 and 7.5 of this Manual identify a series of tests to verify that compliance software accurately demonstrate compliance. A compliance software program vendor may propose alternate tests when the vendor believes that one or more of the standard tests are not appropriate for the compliance software program under consideration. The Commission will evaluate the alternate tests and will accept them if they are found to reflect acceptable engineering techniques.

If alternate tests are accepted by the Commission, the tests will be available for use by all compliance software programs. An alternate test will coexist with the standard test presented in this Manual until the Manual is revised. When a new version of this Manual is produced, the alternative test may be substituted for the current test or may continue to coexist with the original test.

# 65	Residential Energy Compliance Software Programs

## 6.1	General Requirements

Compliance based on simulated energy performance requires that a proposed residence (Proposed Design) be shown to have an annual normalized, modified energy load that is less than or equal to 80% of the annual energy load of the Standard Reference Design as specified in Normative Appendix B, Section B-1.1.1 to make the energy code 20 percent more stringent than the “2007” (Effective October 31, 2007) Florida energy code’s Standard Reference Design (Baseline) features.  The Standard Reference Design and Proposed Design utilized by the energy simulation tool shall be configured and analyzed as specified in Table B-1.1.2 of the Florida Building Code, Energy Conservation.

In accordance with Section 401.2 of the energy code, cCompliance software programs shall designate that the mandatory code provisions automatically perform a variety of functions including those described in Sections 401, 402.4, 402.5, and 403.1, 403.2.2, 403.2.3, and 403.3 through 403.9 (referred to as the mandatory provisions) shall also be met as well as any relevant performance criteria in and Section 405 (performance).

Compliance with this section requires that the mandatory provisions identified in Section 401.2 be met.

The compliance software shall accept a specified range of inputs for the Proposed Design, and then use these inputs to describe the proposed building on the required output forms. The proposed building inputs are also used to create a Standard Reference Design building based on the Proposed Design building and the energy budget generation rules used to incorporate the prescriptive requirements into the Proposed Design.

Certain building descriptors remain the same for both the proposed and standard design but others will change in ways that depend upon the design characteristics, the climate zone, and the prescriptive and mandatory requirements of the energy code.

- The compliance software shall automatically define the Standard Reference Design; determine the proper capacity of the HVAC equipment for the Proposed Design; adjust the HVAC capacity of the Proposed Design in accordance with the reference method; and automatically run the standard reference design to establish the energy budget. [deleted per comment from the 10/17 group meeting; further comment shows this requirement is in the energy code and should be reinstated in the manual]

- The applicant shall perform the tests identified in this Manual to assure that the compliance software program produces results in general agreement with the expected results. These tests verify the implementation of the custom budget procedure, program accuracy and performance relative to the test cases, and acceptable use of calculation inputs

For tests that DO NOT COMPLY, the vendor shall supply diagnostic output that indicates noncompliance and gives the TDV energy information needed to evaluate the test criteria, including the lighting and receptacle portions of the energy budgets for both proposed and standard design. [Work group recommended deletion of this clause; comment by JN]

## 6.2	Minimum Capabilities [new text imported from Section B-1.2.1 of the energy code]

Compliance software programs (energy simulation tools) shall be capable of calculating the annual energy consumption of all building elements that differ between the Standard Reference Design and the Proposed Design and shall include the following capabilities (from B-1.2.1 Minimum capabilities of the energy code).

- Computer generation of the standard reference design using only the input for the proposed design. The calculation procedure shall not allow the user to directly modify the building component characteristics of the standard reference design.

- Calculation of whole-building (as a single zone) sizing for the heating and cooling equipment in the standard reference design residence in accordance with Section 403.6 of the energy code.

- Calculations that account for the effects of indoor and outdoor temperatures and part-load ratios on the performance of heating, ventilating and air-conditioning equipment based on climate and equipment sizing.

- Printed code official inspection checklist listing each of the proposed design component characteristics determined by the analysis to provide compliance, along with their respective performance ratings (e.g., R-value, U-factor, SHGC, HSPF, AFUE, SEER, EF, etc.)

## 6.3	Compliance Report

Documentation verifying the methods and accuracy of the compliance software program tools shall conform to the provisions of this section and be provided to the building code official. Compliance software program provisions and overall stringency shall be as described in Normative Appendix B of the energy code.

The compliance software program tools shall generate a Form 405 (see preferred format in Appendix A of this Manual) report that documents that the proposed design complies with Section 405.3 of the energy code. The compliance documentation shall be submitted to the building official before a building permit is issued and shall include the following information:

- Address or other identification of the residence;

- An inspection checklist documenting the building component characteristics of the Proposed Design as listed in Appendix B, Table B-1.1.2(1) of the energy code. The inspection checklist shall show results for both the Standard Reference Design and the Proposed Design, and shall document all inputs entered by the user necessary to reproduce the results;

- Name of individual completing the compliance report; and

- Name and version of the compliance software tool

Exception: Multiple orientations. When an otherwise identical building model is offered in multiple orientations compliance for any orientation shall be permitted by documenting that the building meets the performance requirements in each of the four cardinal (north, east, south and west) orientations.

The building code official shall require the following documents:

- An Energy Performance Level (EPL) Display Card (prefer format in Appendix C) signed by the builder providing the building component characteristics of the Proposed Design shall be provided to the purchaser of the home at time of title transfer.

- Documentation of the component efficiencies used in the software calculations for the proposed building design.

The building official shall require that an EPL Display Card be completed and certified by the builder to be accurate and correct before final approval of the building for occupancy. Florida law (Section 553.9085, Florida Statutes) requires the EPL Display Card to be included as an addendum to each sales contract for both presold and nonpresold residential buildings. The EPL Display Card contains information indicating the energy performance level and efficiencies of components installed in a dwelling unit. The building official shall verify that the EPL Display Card completed and signed by the builder accurately reflects the plans and specifications submitted to demonstrate energy code compliance for the building.

# 6.4	Residential Energy Performance Testing

This section Chapter specifies required capabilities that compliance software will be tested for. All of the required capabilities are described in terms of the capabilities and algorithms of the 2010 Florida Building Code, Energy Conservation.

Compliance software programs shall account for the energy performance effects of all of the features described in the energy code.

The modeling procedures and assumptions described in this chapter apply to both the Standard Reference Design and Proposed Design. The requirements for the standard design include those that compliance software program shall apply to new features, altered existing features, unchanged existing features or all of the above. In order for compliance software program to become approved, it shall, at a minimum, accept all of the required inputs.

## 6.4.1	Vendor Proposed Alternate Testing Procedures

A compliance software vendor may propose alternate tests when the vendor believes that one or more of the standard tests are not appropriate for the compliance software. The Commission will evaluate the alternate tests and will accept them if they are found to reflect acceptable engineering techniques.

If alternate tests are accepted by the Commission, the tests will be available for use by all compliance software programs. An alternate test will coexist with the standard test presented in this Manual until the Manual is revised. When a new version of this Manual is produced, the alternative test may be substituted for the current test or may continue to coexist with the original test. [Criteria moved to section 5 of this Manual]

## 6.4.12	Residential Accuracy Test cases [was 6.1]

A specific version of HERS BESTEST for Florida was developed in request to DOE by the Florida Solar Energy Center (FSEC).  In its request, FSEC noted that the Florida Building Energy –Efficiency Ratings Act of 1993 requires that Florida’s rating system “be compatible with standard federal rating systems...where applicable....”  The relevant proposed federal guidelines (DOE 10 CFR Part 437) will require that energy analysis tools used for energy ratings are tested according to the HERS BESTEST procedure.  In addition, Florida energy analysis software is used for both energy code compliance and energy performance.  [The work group specified that “expected results” be clarified by referring to the “BESTEST”]

The type of software testing used in this manual is based on inter-model comparisons and is one portion of an overall validation methodology that was first developed by NREL in 1983 (Judkoff et al 1983). The method has been further refined since then by NREL and number of European researchers (Bloomfield, Bowman and Lomas, Irving, Judkoff, Judkoff and Neymark 1995a, 2006; Judkoff et al, Lomas). The overall validation methodology consists of three parts:

- Analytical Verification – in which the output from a program, subroutine or algorithm is compared to the results from a known analytical solution for isolated heat transfer mechanism under simple boundary conditions.

- Empirical Validation – in which calculation results from a program, subroutine, or algorithm, is compared to monitored data from a real structure, test cell, or laboratory experiment.

- Comparative testing – in which a program is compared to itself or to other programs. The comparative approach included “sensitivity testing’ and “intermodal comparisons.” [Work group recommended limiting validation to comparative testing as per comment from JN]

This manual uses the comparative testing method.  Comparative testing is when a program is compared to itself or to other programs.

Comparative testing as applied in the HERS Building Energy Simulation Test (HERS BESTEST) (Judkoff and Neymark 1995b) method includes a set of public domain reference programs that have already been subjected to extensive analytical, empirical, and inter-model testing.

### 6.4.2.1	Reference Test Cases

[Comment from P. Fairey dated 10/21/11:  As currently constructed the simulation tool Technical Assistance Manual provides residential software verification tests only for building heating and cooling loads. It does not provide any tests for residential HVAC systems simulation (which are used to project the annual energy use used to determine the energy budget) or for hot water systems energy use modeling or for distribution system efficiency modeling.  Nonetheless, software verification test suites for all of these simulation models are immediately available and are required for software accreditation to meet RESNET and other standards. For your information and use, I attached a copy of the procedures manual for RESNET accreditation as an IECC software tool so that the information can be incorporated into the Florida Technical Assistance Manual.]

Except as follows, the software verification test suite found in section 2.2 of the RESNET document “Procedures for Verification of International Energy Conservation Code Performance Path Calculation Tools” dated September 2007 shall be utilized to verify the accuracy of the program.

- Tier 1 of the HERS BESTEST as described below.

- The Florida-HERS BESTEST auto-generation test shall be used to verify the ability of the software tool to automatically generate the Florida Building Code’s Standard Reference Design Home.

- HVAC tests – These tests verify the accuracy and consistency with which software tools predict the performance of HVAC equipment, including furnaces, air conditioners and air source heat pump.

- Duct distribution system efficiency tests – These tests verify the accuracy with which software tools calculate air distribution system losses. ASHRAE Standard 152 results are used as the basis of acceptance criteria for this test suite.

- Hot water system performance tests – These tests determine the ability of the software to accurately predict hot water system energy use.

The reference test cases are found in the following document:

NREL/TP-550-23124a    “Home Energy Rating System Building Energy Simulation Test for Florida (Florida-HERS BESTEST),” Volume 1 Tier 1 Tests User’s Manual, version August 1997 _ NREL/TP-550-23124a,  The volumes of interest are as follows: Volume 1 Tier 1 Tests User’s Manual, Judkoff, Ron and Joel Neymark,

Volume 2 Tier 1 Tests Reference Results  [Comments from BW on 10/17; work group recommended]

The authors are Ron Judkoff and Joel Neymark

NREL is a national laboratory of the U.S. Department of Energy. The report was developed under Contract No. DE-AC36-83CH10093, Task No. 0796.5503, managed by the Midwest Research Institute.  A copy of the reference test procedures and sample results may be located from:

National Renewable Energy Laboratory

1617 Cole Boulevard

Golden, Colorado 80401-3393

Managed by Midwest Research Institute for the U.S. Department of Energy under contract No. DE-AC36-83CH10093

Prepared under Task No. 0796.5503

Or online at: http://www.nrel.gov/docs/legosti/fy97/23124a.pdf

The following test procedure is used to evaluate and calibrate the proposed compliance software tool.

### 6.4.3	Testing Procedures [was 6.1]

Using the test cases identified in the reference document in 6.4.2 above,

- Simulate the cases as outlined  in the document (Orlando, FL TMY)

- Record the results using the tables identified in the reference test documents

- Compare your output with Volume 2 Tier 1 Tests Reference Results

- Determine if your compliance software pass or fail the tests

A software vendor may also submit test results for other regions as supplementary documentation shall also submit test results for Las Vegas, NV and/or Colorado Springs, CO.  The source and data of the reference test results used for comparison must be submitted with the application.  [Work group comment that other cities may be used to demonstrate compliance]

[Detail Testing Procedures deleted per work group recommendation]

A program may be considered as having passed successfully through the test when its results compare favorably with the reference results (Volume 2 Tier 1 Tests for Orlando, FL).

# 7	COMMERCIAL ENERGY COMPLIANCE SOFTWARE PROGRAMS CONSERVATION

Commercial suites of test include two types of verification.  First is that the compliance software is capable of automatically generating the Standard Reference Design from user inputs.  The Standard Reference Design generated at a minimum shall satisfy the requirements of Section 506 and Table 2.2 of the Normative Appendix B of the energy code.  The second suite of tests involves the calculation of the Total Building Performance of a Proposed Design.

[text of next 4 paragraphs moved here from 7.1]

The commercial requirements of this Manual are largely based on criteria in COMNET, the Commercial Buildings Energy Modeling Guidelines and Procedures, RESNET Publication 2010-001. The manual is consistent with ASHRAE Standard 90.1-2007, but does not include addenda, approved or otherwise. ASHRAE Standard 90.1 is under continuous maintenance. This means that the committee regularly develops, approves and publishes addenda to the standard. At the time of this writing, a number of addenda to Standard 90.1 have been approved and an even greater number is pending. ASHRAE intends to gather these addenda and include them in the 2010 publication of the standard. The COMNET manual will be updated when the 2010 publication of Standard 90.1 is released.

The modeling rules and procedures in the COMNET this manual are consistent with the Performance Rating Method (PRM) in Appendix G of ASHRAE Standard 90.1-2007. This method of testing is provided for analyzing and diagnosing building energy simulation software using software-to-software and software-to-quasi-analytical-solution comparisons. The methodology allows different building energy simulation programs, representing different degrees of modeling complexity, to be tested by comparing the predictions from other building energy programs to the simulation results provided by the rating software in question.  Energy code compliance software programs shall utilize auto-generation files specific to Chapter 11 of ASHRAE 90.1, not Appendix G of the standard.

The tests in this manual are intended to be used for buildings that are in the planning, design or construction phases. It is also intended to apply to buildings that are within the scope of ASHRAE Standard 90.1-2004. The long-term goal of this manual is to define modeling rules and procedures for all conceivable design features that may be incorporated in buildings. However, this goal cannot be fully achieved due to limitations in the development of energy simulation algorithms, and due to the natural lag time between the introduction of an advanced energy efficiency measure or device and the development of algorithms to simulate its performance.

The goal of the manual is to provide methods that are as flexible and accurate as possible. This goal can best be achieved if the manual is a ‘living document,’ changing and growing as increasing amounts of information and better modeling methods become available.

7.1 Vendor Propose Alternate Testing Procedures

A compliance software vendor may propose alternate tests when the vendor believes that one or more of the standard tests are not appropriate for the compliance software. The Commission will evaluate the alternate tests and will accept them if they are found to reflect acceptable engineering techniques.

I alternate tests are accepted by the Commission, the tests will be available for use by all compliance software programs. An alternate test will coexist with the standard test presented in this Manual until the Manual is revised. When a new version of this Manual is produced, the alternative test may be substituted for the current test or may continue to coexist with the original test.  [Section moved to 5; don’t need to repeat it]

## 7.12	General Requirements [Recommend make this 7.1]

Compliance software tools shall be capable of calculating the annual energy consumption of all building elements that differ between the standard reference design and the proposed design and shall include the following capabilities. [text moved here from 7.1, Calculation Software Tools, by comment from JN; recommended by group]

- Computer generation of the standard reference design using only the input for the proposed design. The calculation procedure shall not allow the user to directly modify the building component characteristics of the standard reference design.

- Building operation for a full calendar year (8760 hours).

- Climate data for a full calendar year (8760 hours) and shall reflect approved coincident hourly data for temperature, solar radiation, humidity and wind speed for the building location.

- Ten or more thermal zones.

- Thermal mass effects.

- Hourly variations in occupancy, illumination, receptacle loads, thermostat settings, mechanical ventilation, HVAC equipment availability, service hot water usage and any process loads.

- Part-load performance curves for mechanical equipment.

- Capacity and efficiency correction curves for mechanical heating and cooling equipment.

- Printed building code official inspection checklist listing each of the proposed design component characteristics from Table 506.5.1(1) determined by the analysis to provide compliance, along with their respective performance ratings (e.g., R-value, U-factor, SHGC, HSPF, AFUE, SEER, EF, etc.).

In accordance with Section 501.2 of the energy code, compliance software programs shall designate that the mandatory code provisions Compliance with this section requires that the mandatory and applicable prescriptive criteria of Sections 502, 503, 504 and 505 of the energy code shall also be met as well as any relevant performance criteria in Section 506.

### 7.12.1	Trade-Offs Limited to Building Permit

When the building permit being sought applies to less than the whole building, only the calculation parameters for that part of the building related to the systems to which the permit applies shall be allowed to vary.  Where an existing building and addition are calculated to determine compliance in accordance with Section 101.4.3 of the energy code and one or more existing components are unable to meet current prescriptive energy code minimum requirements, said component(s) shall be modeled in their existing (or replaced) condition need not meet energy code if the entire building is brought into compliance with the energy code. Future building components shall meet the prescriptive requirements of 502, 503, 504 or 505, of the Energy Code, as applicable.

### 7.2	Requirements specific to credit options [Delete section]

Compliance calculation for technologies that meet the criteria for various options specified in section 506.3 of the energy code shall utilized criteria in section 506.3. [This section is not needed; Covered by new 7.1 …”as well as any relevant performance criteria in Section 506”]

## 7.26	Performance-based Compliance [recommend making this section 7.2 after General requirements]

Compliance based on total building performance requires that a proposed building (Proposed Design) be shown to have an annual energy cost that is less than or equal to the annual energy cost of the Standard Reference Design. Energy prices shall be taken from a source approved by the Florida Building Commission. Nondepletable energy collected off site shall be treated and priced the same as purchased energy.  Energy from nondepletable energy sources collected on site shall be omitted from the annual energy cost of the Proposed Design.  Refer to Section 506 of the energy code;   Normative Appendix B section B-2.1 states that the Standard Reference Design totals shall be adjusted by a factor of 0.8.

This section establishes criteria for compliance using total building performance. It may be employed for evaluating the compliance of all proposed designs, except designs with no mechanical system.  The following systems and loads shall be included in determining the total building performance:  heating systems, cooling systems, service water heating, fan systems, lighting power, receptacle loads and process loads.

### 7.26.1	Calculation Procedure

The standard reference design and proposed design shall be configured and analyzed as specified by Table B-2.2 of the Energy Code. Except as specified by this section of the Energy Code, the standard reference design and proposed design shall be configured and analyzed using identical methods and techniques. The Standard Reference Design totals for the Total Building Performance compliance method shall be adjusted by a factor of 0.80 to make the energy code 20 percent more stringent than the “2007” Florida energy code’s Standard Reference Design features.

The compliance software program shall be capable of modeling:

- The Standard Reference Design building systems defined in 2010 Florida Building Code.

- The lighting, water heating, HVAC and miscellaneous equipment detailed in the Standards.

- All compulsory and required features as listed in Normative Appendix B and Section 506 of the energy code.

- The capability to model multiple zone systems shall allow at least 10 thermal blocks to be served by one multiple zone system.

- The compliance software program shall be capable of modeling plenum air return.

### 7.26.2	Calculation Methods

The compliance software program shall calculate the annual consumption of all end uses in buildings, including fuel and electricity for:

- HVAC (heating, cooling, fans, and ventilation);

- Lighting (both interior and exterior);

- Receptacles and miscellaneous electric;

- Service water heating;

- Process energy uses;

- Commercial refrigeration systems; and

- All other energy end uses that typically pass through the building meter

The compliance software program shall perform a simulation on an hourly time interval (at a minimum) over a one year period (8760 hours) with the ability to model changes in weather parameters, schedules, and other parameters for each hour of the year. This is typically achieved by specifying a 24-hour schedule for each day of the week plus holidays.

### 7.26.3	Error Handling

The software shall identify error conditions, prevent completion of the Compliance analysis, and provide information to the user describing the error that has occurred and what steps the user should take to remedy the situation.

### 7.26.4	Climate Data

The compliance software program shall perform simulations using hourly values of climate data, such as temperature and humidity, derived from WYEC (Weather Year for Energy Calculation) or TMY (Typical Meteorological Year) climate data. The compliance software rogram shall calculate solar radiation on exterior surfaces on an hourly basis from the values of direct normal irradiance and diffuse horizontal irradiance contained in the climate data, taking ground reflectance into account.

Climate criteria for the performance-based code compliance methods are determined by climate data from the weather data collection station nearest to the building’s location.  All Florida counties are considered warm humid counties.

### 7.26.5	Utility Rates

The compliance software program shall be capable of simulating time-of-use rates and apply both demand and energy charges for each time period of the rate schedule.  Rates are available from the Florida Public Service Commission, http://www.floridapsc.com/Default.aspx

[Work group agreed with comment from WW: use Table 2-1 without accompanying references]

## 7.34	Managing User Input

This section addresses the processes of data entry and the validation of user input data that can be performed prior to and independent of the energy simulation.

### 7.34.1	Building Descriptor Inputs and Restrictions

Building descriptors are discussed in Section 506 and listed in tabular form in Normative Appendix B of the energy code.  All inputs shall conform to the input conditions and restrictions specified in Normative Appendix B and Section 506 of the energy code.

Four levels of restriction are specified for building descriptors. The most limiting restriction is a prescribed value. This is an input that must be used in all instances, with no variation. A critical default may be overridden, but when it is, the user must provide special documentation. A default is provided for convenience and may be overridden by the user with no special documentation. For many inputs there is no restriction.

Restrictions apply to all required inputs. If the software provides a means for the user to input building descriptors listed as optional in Normative Appendix B (of the energy code), all input conditions and restrictions in Normative Appendix B and Section 506 (of the energy code) pertaining to those building descriptors shall be met.

### 7.34.2	User Interface

The software user interface shall:

- clearly indicate when a building descriptor has an associated default,

- indicate what the default value is, and

- provide a convenient means for the user to over-ride the default. When critical default values are overridden, the software interface shall notify the user that documentation of the revised assumption is required.

The software is not required to provide a means for users to enter data for building descriptors designated as prescribed in Normative Appendix B  and Section 506 (of the energy code). However, if the user is permitted to input values for prescribed inputs, the software must inform the user that a prescribed value and not the value input by the user will be used in the compliance.

No restrictions are specified for unsanctioned inputs. If the software uses unsanctioned inputs, the software documentation or help system shall specify the applicability of the building descriptors, its definition, the units in which it is expressed, restrictions on input for the proposed design, and, if applicable, how the building descriptor is defined for the Standard Reference Design building.

Compliance software programs may not provide default assumptions other than those specified in Normative Appendix B and Section 506 (of the energy code). However, the software may assist the user in describing the proposed design by displaying typical values for building descriptors, provided deliberate action by the user is necessary before a displayed value is used.

### 7.34.3	Compulsory Input Checks

The software shall check to ensure that valid entries have been made for all compulsory building descriptors before the user is permitted to proceed with the next step in the compliance process. Normative Appendix B and Section 506 (of the energy code) specify the compulsory building descriptors.

### 7.34.4	Handling of Missing Inputs

If a required input is missing or invalid, the software shall:

- notify the user that the input is missing or invalid,

- identify the input field(s) with missing or invalid data, and

- prevent the user from moving to the next step of the Compliance process.

The software may provide additional information designed to help the user correct the deficiency.

### 7.34.5	Validity Checks

The software shall check all user inputs to ensure that the following conditions are met:

- Simulation Tool Limits- Inputs do not exceed the minimums or maximums for the parameters permitted by the simulation engine.

- Compliance Rule Limits- Inputs do not exceed minimums or maximums for the descriptors specified in the Energy Codes.

- Simulation Tool Discrete Options- Inputs correspond with valid discrete or list options for parameters available in the simulation engine.

- Compliance Rule Discrete Options- Inputs correspond with valid discrete options provided for in Energy Codes.

### 7.34.6	Handling Invalid Input

When invalid data is entered, the software shall:

- notify the user of the invalid input,

- identify the nonconforming input field, and

- prevent execution of the next step of the Compliance process

The software may provide additional information designed to help the user correct the deficiency.

### 7.34.7	Consistency Checks

The consistency checks described above are intended to identify errors and oversights in user input and thereby help ensure that the building description is complete and interpretable by the energy analysis program. Examples of consistency checks include that window should not exceed the areas of wall in which they are contained and that the necessary plant equipment has actually been connected to the secondary HVAC systems. The software may include additional consistency checks provided these additional consistency checks are clearly documented in the user documentation or on-line help.

### 7.34.8	Handling Inconsistent Input

If the proposed design fails a consistency check, the software shall:

- notify the user that an inconsistency exists,

- identify the specific consistency check that has been failed,

- identify the inconsistent input fields, if feasible, and

- prevent execution of the next step of the Compliance process

The software may provide additional information designed to help the user correct the deficiency.

## 7.45	Documentation

Compliance software tools shall be utilized to conform to the provisions of this section.  Compliance software provisions and overall stringency shall be as described in Normative Appendix B of the energy code.

Compliance documentation includes the forms, reports and other information that is submitted to the building department with an application for a building permit. The purpose of the compliance documentation is to enable the plans examiner to verify that the building design complies with the code Standards and to enable the field inspector to readily identify building features that are required for compliance.

### 7.45.1	Compliance Report

The compliance software tools shall generate a Form 506 report (see preferred format in Appendix B) that documents that the proposed design has annual energy costs less than or equal to the annual energy costs of the standard reference design. By standardizing the reports, all compliance authorities will be able to view the same building information and evaluate the project for certification. The required formats for building information reports are electronic (PDF) and hard copy. The hard copy standard reports will be in PDF (Portable Document File). Both report formats will be automatically generated by the software. Each page of the report will have a header with the project name and date.

The compliance documentation shall be submitted to the building code official before a building permit is issued and shall include the following information:  [Text moved here from 7.7.1, Content and Format of Code Reports]

- Address of the building (Section 506.4.1 of the energy code);

- An inspection checklist documenting the building component characteristics of the proposed design as listed in Table B-2.2 of Appendix B of the Energy Code. The inspection checklist shall show the estimated annual energy cost for both the standard reference design and the proposed design;

- Name of individual completing the compliance report; and

- Name and version of the compliance software tool.

5.	Building information should be presented in four standard reports:

- Building Summary

- Energy Measures

- Energy Results

- Representations

### 7.45.2	Additional documentation (Section 506.4.2)

The building code official shall require the following documents:

- Thermal zoning diagrams consisting of floor plans showing the thermal zoning scheme for the proposed design.

- Input and output report(s) from the energy analysis simulation program containing the complete input and output files, as applicable. The output file shall include energy use totals and energy use by energy source and end-use served, total hours that space conditioning loads are not met and any errors or warning messages generated by the simulation tool as applicable;

- An explanation of any error or warning messages appearing in the simulation tool output; and

- A certification signed by the design professionals responsible under Florida law for the design of lighting, electrical, mechanical, and plumbing systems and the building shell providing the building component characteristics of the proposed design as given in Table B-2.2 of Appendix B of the energy code.  See Section 103.1 of the Florida Building Code.

## 7.53	Commercial Tests Verification [was 7.1, Software Requirements]

This section chapter contains the software requirements that must be implemented by approved compliance software programs. The tests fall into the following categories:

    - Tests to verify that the software is evaluating thermal loads and the response of the HVAC systems to these loads in a manner that is acceptable. These tests reference ASHRAE Standard 140-2007, Standard Method of Test for Evaluation of Building Energy Analysis Computer Programs. [work group recommended ASHRAE 140-2007]

- This method of testing is provided for analyzing and diagnosing building energy simulation software using software-to-software and software-to-quasi-analytical-solution comparisons. The methodology allows different building energy simulation programs, representing different degrees of modeling complexity, to be tested by comparing the predictions from other building energy programs to the simulation results provided by the Compliance Software in question. [Text moved here from ASHRAE 140-2007 below for consistency]

- COMNET compliant software is required to perform the ASHRAE Standard 140-2007 suite of software tests and the results of these tests shall conform to the COMNET acceptance requirements [Commercial Buildings Energy Modeling Guidelines and Procedures, RESNET Publication 2010-001]. All tests shall be completed in accordance with the requirements of ASHRAE Standard 140-2007. The resulting estimates of energy consumption shall fall between the minimum and maximum values established by COMNET, unless a logical explanation is provided using the standard output report block for “anomalous results” provided with Standard 140-2007 Addendum A. The portfolio folder for Appendix E contains spreadsheets wherein the software vendor enters the results of the Standard 140 simulations for comparison against the criteria. When results from candidate software fall outside the COMNET acceptance range or when candidate software is unable to perform one of the tests, the vendor shall provide an explanation of the reason as per ASHRAE Standard 140-2007 Addendum A requirements. The portfolio folder for Appendix E also contains a methodology paper that describes how the COMNET acceptance criteria were developed.

- A PDF copy of the document can be downloaded from the following URL: http://www.comnet.org/mgp/content/31-ashrae-standard-140-2007-tests

    - Tests to verify that the candidate building or the proposed design is modeled with the correct fixed and restricted inputs, including schedules of operation, receptacle loads, process loads and other components.

    - Tests to verify that the Standard Reference Design building is created correctly, e.g. that the Standard Reference Design HVAC system is properly specified and that other components of the Standard Reference Design are correctly defined as per Table B-2.2 of Normative Appendix B of the energy code.

    - Reference building auto-generation test – use test cases specific to Florida necessary to verify that the software automatically generates an accurate standard reference design as contained in Section B-2.2 of the energy code given only the building information from the proposed design. [Work group on 10/17/11 recommended using required criteria of Section B-2.2 of the Florida Building Code, Energy Conservation for auto-generation testing of standard reference design]

    - Test result format.  COMNET spreadsheets and reports may be used to demonstrate test results. 7.7.1 Content and Format of Code Reports. The establishment of these reports will standardize the way energy modeling output data is presented for certification of compliance software programs to various Compliance authorities. By standardizing the reports, all Compliance authorities will be able to view the same building information and evaluate the project for certification. Four spreadsheets are provided in Appendix E of the publication Commercial Buildings Energy Modeling Guidelines and Procedures for documenting results from candidate software and comparing to the COMNET acceptance ranges. These parallel the spreadsheets provided with ASHRAE Standard 140-2007. For each of the spreadsheets, the fields where data is entered are shaded pale yellow. The tabs where data is entered and where results are reviewed are shown in Table 3.2.1-1 [8] of that document. [Some original text from 7.1, Software Requirements]

A program may be thought of as having passed successfully through the test when its results compare favorably with the reference program outputs.

Test Report Overview  [moved to 7.4, Documentation]

The required formats for building information reports are electronic (PDF) and hard copy. The hard copy standard reports will be in PDF (Portable Document File). Both report formats will be automatically generated by the software. Each page of the report will have a header with the project name and date.

Content

Building information will be organized into four standard reports:

1)Building Summary

2)Energy Measures

3)Energy Results

4)Representations

8.1 References

ANSI/ASHRAE Standard 140-2007. Standard Method of Test for the Evaluation of Building Energy Analysis Computer Programs. (2007). Atlanta, GA: American Society of Heating, Refrigerating, and Air-Conditioning Engineers.

Judkoff, R., and J. Neymark. (1995a). International Energy Agency Building Energy Simulation Test (BESTEST) and Diagnostic Method. NREL/TP-472-6231. Golden, CO: National Renewable Energy Laboratory. http://www.nrel.gov/docs/legosti/old/6231.pdf (PDF 13.8 MB)

Judkoff, R., and J. Neymark. (1995b). Home Energy Rating System Building Energy Simulation Test (HERS BESTEST). NREL/TP-472-7332. Golden, CO: National Renewable Energy Laboratory. http://www.nrel.gov/docs/legosti/fy96/7332a.pdf (PDF 5.6 MB)

http://www.nrel.gov/docs/legosti/fy96/7332b.pdf (PDF 1.9 MB)

Judkoff, R.; Neymark, J. (2006). Model Validation and Testing: The Methodological Foundation of ASHRAE Standard 140. ASHRAE Transactions: Papers Presented at the 2006 Annual Meeting, 24-28 June 2006, Quebec City, Canada. Atlanta, GA: American Society of Heating, Refrigerating and Air-Conditioning Engineers, Inc. (ASHRAE) Vol. 112, Pt. 2: pp. 367-376; NREL Report No. CP-550-41015. [work group recommended only include this reference for informational purposes.]

Judkoff, R., D. Wortman, B. O'Doherty, and J. Burch. (1983). A Methodology for Validating Building Energy Analysis Simulations. SERI/TR-254-1508. Golden, CO: Solar Energy Research Institute (now National Renewable Energy Laboratory).

Neymark, J.; Girault, P.; Guyon, G.; Judkoff, R.; LeBerre, R.; Ojalvo, J.; Reimer, P. (2004). ETNA BESTEST Empirical Validation Test Specification. Golden, Colorado: J. Neymark & Associates; Moret sur Loing, France: Electricité de France. In collaboration with National Renewable Energy Laboratory, Golden, Colorado.

Neymark, J., and R. Judkoff. (2002a). International Energy Agency Building Energy Simulation Test and Diagnostic Method for Heating, Ventilating, and Air-Conditioning Equipment Models (HVAC BESTEST) Volume 1: Cases E100-E200. NREL/TP-550-30152. Golden, CO: National Renewable Energy Laboratory. http://www.nrel.gov/docs/fy02osti/30152.pdf (PDF 4.5 MB)

Neymark, J.; Judkoff, R. (2004). International Energy Agency Building Energy Simulation Test and Diagnostic Method for Heating, Ventilating, and Air-Conditioning Equipment Models (HVAC BESTEST) Volume 2: Cases E300–E545. NREL/TP-550-36754. Golden, CO: National Renewable Energy Laboratory.

http://www.nrel.gov/docs/fy05osti/36754.pdf (PDF 15.4 MB)

California Energy Commission’s  Residential Alternative Calculation Method (AC M) Approval Manual, 2008 Building Efficiency Standards, CEC-400-2008-002-CMF.

http://www.energy.ca.gov/2008publications/CEC-400-2008-002/CEC-400-2008-002-CMF.PDF

California Energy Commission’s  Non-Residential Alternative Calculation Method (AC M) Approval Manual, 2008 Building Efficiency Standards, CEC-400-2008-003-CMF.

http://www.energy.ca.gov/2008publications/CEC-400-2008-002/CEC-400-2008-003-CMF.PDF

State of Florida Building Commission, 2010 Florida Building Codes, Energy Conservation. http://www.floridabuilding.org/c/default.aspx

COMNET, Commercial Buildings Energy Modeling Guidelines and Procedures, RESNET Publication 2010-001, August 16, 2010

http://www.comnet.org/mgp/content/commercial-buildings-energy-modeling-guidelines-procedures-mgp

ASHRAE’s Standard 90.1-2010 User's Manual. American Society of Heating, Refrigerating and Air-Conditioning Engineers / 2011 /  ISBN: 9781933742960

http://www.ashrae.org/publications/page/90-1usersmanual

Procedures for Verification  of International Energy Conservation Code Performance Path Calculation Tools.  RESNET Publication No. 07-003. March 2007

http://resnet.us/

Forms 405, 506 and EPL Card Templates.  Florida Solar Energy Center, Cocoa Beach, Fl.

EnergyGauge USA: Code Compliance and Home Energy Rating Software, http://www.energygauge.com/usares/default.htm

EnergyGauge Summit FlaCom,

http://www.energygauge.com/flacom/default.htm

Appendix A Procedure for Developing Example Pass/Fail Criteria [Work group recommended taking out this appendix; not needed]

# APPENDIX A

# RESIDENTIAL FORM 405

# PREFERRED CONTENT AND FORMAT-

In accordance with Section 405.4 of the Florida Building Code, Energy Conservation, the printout from computer programs approved by the Florida Building Commission for use as code compliance Form 405-2010 for residential applications shall contain all information required to determine code compliance for low-rise residential buildings, to include but not be limited to the following information.  Compliance software program printout Form 405s should be consistent with the format described below. Associated forms for air infiltration and duct testing are advised.

A.1  An Administrative page of the printout should contain the following information:

- Form title and headings:

  - Form 405-2010

  - Florida Building Code, Energy Conservation

  - Residential Simulated Performance Alternative

- Project information box

  - Project name

  - Street address/city/state/zip

  - Owner

  - Design location

  - Builder name

  - Permit office

  - Permit number

  - Jurisdiction

- Summary of building components and features

  - New construction or existing

  - Single- or multiple-family

  - Number of units, if multiple family

  - Number of bedrooms

  - Whether it is a worst-case calculation

  - Window U-factor. SHGC and area for all windows in the building

  - Floor type, insulation R-value and area (or perimeter if slab)

  - Wall type, insulation R-value and area by type of wall

  - Ceiling types, insulation R-value and area by type of ceiling

  - Duct location, R-value and type for supply, return and air handler

  - Cooling system type, capacity and efficiency

  - Heating system type, capacity and efficiency

  - Hot water system type, capacity and efficiency

  - Any conservation credits provided in the calculation per S. 405.6 of the code.

- Pass/Fail box

  - Percent glass to conditioned floor area

  - Total Proposed Design loads

  - Total Standard Reference Design loads

  - Whether the building Passes or Fails code compliance

- Compliance certification box

  - Statement, signature and date by the individual completing the compliance report as follows:

    - Statement: “I hereby certify that the plans and specifications covered by this calculation are in compliance with the Florida Building Code, Energy Conservation.”

      - PREPARED BY:___________________

      - DATE:__________________________

  - Statement, signature and date by the owner of the building

    - Statement: “I herby certify that this building, as designed, is in compliance with the Florida Building Code, Energy Conservation.”

      - OWNER/AGENT:___________________

      - DATE:___________________________

  - Statement, signature and date by the code official reviewing the plans and compliance report:

    - Statement:  “Review of the plans and specifications covered by this calculation indicates compliance with the Florida Building Code, Energy Conservation. Before construction is completed, this building will be inspected for compliance with Section 553.908, Florida Statutes.”

      - BUILDING OFICIAL:________________

      - DATE:___________________________

- Name and version of the compliance software tool

A.2  Description of the building. Input Data to be consistent with the plans may include, but not be limited to:

- Project information

- Climate zone information by design location

- Floor type, materials, area or perimeter, R-value

- Roof type, materials area, solar absorptance, testing radiant barrier system, pitch, other relevant information as required by code

- Ceiling type, materials, R-value, area, truss type, framing fraction

- Wall type, orientation, whether it is exterior or adjacent, R-value, area, sheathing, framing fraction solar absorptance

- Door  type, orientation U-value, area

- Window type(s), orientation, U-factor, SHGC, area, overhang, separation, interior shading, screening or storm windows

- Infiltration of the building (SLA, CFM, ACH, ELA, EqLA) and forced ventilation of the building (supply CFM, exhaust CFM, run time, fan watts)

- Garage area, insulation R-value if conditioned

- Cooling system type, efficiency, capacity, air flow, SHR, other relevant information

- Heating system type, efficiency, capacity, other relevant information

- Hot water system type, efficiency, capacity, any code-required credit requirements

- Duct and air handler location, R-value, area, leakage type and percent, Qn, RLF

- Thermostat type and setting

- Ceiling fan use, if applicable

A.3 Code Compliance Checklist may include, but not be limited to:

- Form name, compliance method

- Address and permit number of building permitted

- Infiltration reduction compliance summary checklist by component, code section, code requirements  for said component and a space to be checked for code compliance for the following components:

  - Exterior windows & doors

  - Exterior & adjacent walls

  - Floors

  - Ceilings

  - Recessed lighting fixtures

  - Multiple-story house requirements

  - Any other infiltration requirements

- Other prescriptive measures checklist by component, code section and summary of requirement(s)

  - Water heaters requirements

  - Swimming pool & Spa requirements

  - Shower heads

  - Air distribution system installation/insulation requirements

  - HVAC control requirements

  - Ceiling and common wall insulation minimums

  - Lighting equipment requirements

A.4 Associated residential forms

- An EPL Display Card shall be printed from the compliance software program that contains the information on and is formatted as is the EPL Display Card contained in Appendix C of the energy code.

- Air barrier and insulation inspection component criteria

- Air distribution system test report

- Envelope leakage test report

- Form 402-2010, the Residential Building Thermal Envelope Approach, may be printed out as a function of the compliance software program for all applications including new residential buildings, additions to residential buildings, and new building systems installed in existing buildings.

- Alternate Form 402, the Total UA Alternative, may be printed out from the compliance software program if included in the software.

EXAMPLE Residential Form 405-2010, EPL Display Card and other code-required form printouts

# APPENDIX B

# FORM 506-2010

# COMMERCIAL and HIGH_RISE RESIDENTIAL

# PREFERRED CONTENT AND FORMAT-

In accordance with Section 506.4.1 of the Florida Building Code, Energy Conservation, the printout from computer programs approved by the Florida Building Commission for use as code compliance Form 506-2010 for commercial and high-rise residential applications shall contain all information required to determine code compliance for said buildings, to include but not be limited to the following information.  Compliance software program printout Form 506s should be consistent with the format described below. should contain, but not be limited to, the following information and be consistent with the format described below.

B.1  An Administrative page of the printout should contain the following information:

- Form title and headings:

  - Form 506-2010

  - Florida Building Code, Energy Conservation

  - Total Building Performance for Commercial and High-Rise Residential Buildings

- Project information box

  - Project name

  - Street address/city/state/zip

  - Type of building (occupancy)

  - Class of building (new, renovation, etc)

  - Conditioned floor area

  - Number of stories

  - Owner

  - Design location

  - Builder name

  - Permit office/jurisdiction

  - Permit number

B.2 Output project data

- Climate zone information by design location

- Building End Uses: Proposed design vs Standard Reference Design

  - Electricity

  - Area lights

  - Miscellaneous equipment

  - Pumps & miscellaneous

  - Space cooling

  - Vent fans

  - Natural gas

  - Space heat

- External lighting compliance

- Lighting controls compliance

- System report compliance

- Plant compliance

- Water heater compliance

- Piping system compliance

- Other required compliance requirements

B.3  Compliance Summary

- Whether the building Passes or Fails code compliance

  - Gross Energy Cost (in dollars)

  - Lighting controls

  - External lighting

  - HVAC system

  - Plant

  - Water heating systems

  - Piping systems

  - Inspection checklist

- Any conservation credits provided in the calculation per S. 506.3.3 of the code

- Compliance certification box

  - Statement, signature and date by the individual completing the compliance report as follows:

    - Statement: “I hereby certify that the plans and specifications covered by this calculation are in compliance with the Florida Building Code, Energy Conservation.”

      - PREPARED BY:___________________

      - DATE:__________________________

  - Statement, signature and date by the owner of the building

    - Statement: “I herby certify that this building, as designed, is in compliance with the Florida Building Code, Energy Conservation.”

      - OWNER/AGENT:___________________

      - DATE:___________________________

  - Where Florida law requires a design to be performed by a registered design professional, said design professional shall certify compliance of building by signing and providing their registration number:

    - Architect:_________________ __  Registration No.:__________

    - Electrical Designer:_________ ___ Registration No:__________

    - Lighting Designer:______________Registration No:__________

    - Mechanical Designer:______ _____Registration No:__________

    - Plumbing Designer:________ ____ Registration No:__________

  - Statement, signature and date by the code official reviewing the plans and compliance report:

    - Statement:  “Review of the plans and specifications covered by this calculation indicates compliance with the Florida Building Code, Energy Conservation. Before construction is completed, this building will be inspected for compliance with Section 553.908, Florida Statutes.”

      - BUILDING OFICIAL:________________

      - DATE:___________________________

- Name and version of the compliance software tool

B. 4  Input data.  Description of the building; data to be consistent with the plans may include, but not be limited to:

- Project information

  - Zones

  - Spaces

  - Lighting

  - Walls

  - Windows

  - Doors

  - Roofs

  - Skylights

  - Floors

  - Systems

  - Plant

  - Water heaters

  - Exterior lighting

  - Piping

  - Fenestration used

  - Materials used

  - Constructs used

B.5 Code Compliance Checklist

- Form name, compliance method

- Address and permit number of building permitted

- Other prescriptive measures checklist by component, code section and summary of requirement(s)

  - Operations manual

  - Air infiltration

  - Dehumidification

  - HVAC efficiency

  - HVAC controls

  - Ventilation

  - Piping insulation

  - Duct insulation and design

  - Balancing

  - Water heaters requirements

  - Swimming pools

  - Shower heads

  - Lighting controls

B.6   Other forms that may be printed out from the commercial compliance software program:

- Form 502-2010 for either shell buildings or renovations/new building systems.

EXAMPLE COMMERCIAL and HIGH-RISE RESIDENTIAL Form 506-2010 Printouts