To:		Chairman Ed Carson and 				Date:	August 10, 2009

Members Product Approval POC

From:		Theodore Berman, P.E. , Manager

Ted Berman and Associates, LLC

Product Approval System Administrator

Subject:	Administrative Validation of Application FL12017

Description of Product:

FL12017 was an application proposed to comply with the 2007 Florida Building Code for a product in the Category Shutters and Subcategory Roll-up.  The assembly on the application consisted of aluminum slats supported by fixed walls and or mullions and reinforced when necessary with storm bars.

The application was made using the Evaluation Report by Professional Engineer Method.  The evaluator was a third party independent FL PE and the application accepts the validation using the Administrative Validation checklist.

The evaluation report dated January 8, 2009 indicates:  “The shutters, stormbars and header were impacted and qualified to resist large missile impact under sections 1609.1.2 of the Florida Building Code, as per ASTM 1886 & 1996 Protocols.”

The Installation Instructions referred on the evaluation report shows mullions up to the following heights:

2” X 6”   0.125” wall thickness up to 19’-8” height

2” X 6”  0.250”  wall thickness up to  22-7” height

and storm bars 2” X 4” 0.125” wall thickness up to 16’-7” height.

A review of the test reports indicates that only a configuration with 2” X 6” 0.125” wall thickness up to 10’-0” height was tested.  This is the only vertical support element tested for tests performed prior to date of evaluation report and validation on or about January and February 2009.

The Administrative Validation requires Validator to verify the following:

5.	Verify that test reports demonstrating compliance were performed at a test lab accredited by an approved accreditation body for the test performed, and verify test lab location.

6.	Verify that the test reports or rational analysis are required by the Building Code as a component of a product approval for the product category and subcategory.

8.	Verify that all data, test reports and calculations, are substantiated on the report.

Administrative Validation of Application FL12017

August 10, 2009

Page 2

Question 1.	Were the validations checklist indicating check marks for Items 5, 6 and 8 dated January 9, 2009 and February 11, 2009 compliant with the requirements of the Administrative Validation for checklist items 5, 6 and 8?

Question 2.	If any of the answers to Question 1 is negative, is the Validator subject to disciplinary action for violation of Section 9B-72.080 (4) (c) FAC?

9B-72.080 Product Validation by Approved Validation Entity for  State Approval.

(c) Violation of subsection 9B-72.080(1) or (2) or (3) F.A.C. Verifying, validating or certifying by using fraud, deceit, misleading, or untrue representations of items on validation checklist set forth in subsection 9B-72.130(3), F.A.C., for which the Commission shall impose a minimum sanction of a $5,000 fine, or probation, or suspension, or any combination thereof; and a maximum sanction of a $10,000 fine and revocation.