“EXHIBIT B”

SECTION 1609 WIND LOADS

1609.1 Applications. Buildings, structures and parts thereof shall be designed to withstand the minimum wind loads prescribed herein. Decreases in wind loads shall not be made for the effect of shielding by other structures. 

All exterior wall coverings and soffits shall be capable of resisting the design pressures specified for walls for components and cladding loads in accordance with Section 1609.1.1.

Where reference is made to Figure 1609 State of Florida Debris Region and Basic Wind Speed for purposes of design basis, the specific information from Figure 1609.1 Basic Wind Speed Map of Palm Beach County shall be utilized as authorized in Note 2 of Figure 1609. Basic Wind Speeds for Palm Beach county are :

Projected Wind Zones : 	The three separate wind speed zones in PBC:

Zone		Location in Palm Beach County 				3-second gust

140 * 		East of the 130 mph line 						140 mph

130 * 		West of 130 mph line to 120 mph line 				130 mph

120 * 		West of 120 mph line 						120 mph

* Exposure Category shall be determined per FBC 1609.4