Changes to Subchapter 13-1

[Change Section 13-101.5.7 to read as follows:]

13-101.5.7 Any building where heating or cooling systems are provided which are designed for purposes other than general space comfort conditioning. Buildings included in this exemption include:

1.     Buildings containing a system(s) designed and sold for dehumidification purposes only and controlled only by a humidistat. No thermostat shall be installed on systems thus exempted from this code. The provisions of Section 13-413 shall apply.

2.     Commercial service areas where only ceiling radiant heaters or spot coolers are to be installed which will provide heat or cool only to a single work area and do not provide general heating or cooling for the space.

3.     Buildings heated with a system designed to provide sufficient heat only to prevent freezing of products or systems. Such systems shall not provide heating above 50 F (10°C).

4.     Pre-manufactured freezer or refrigerated storage buildings and areas where the temperature is set below 40 F (4°C) and in which no operators work on a regular basis.

5.     Electrical equipment switching buildings which provide space conditioning for equipment only and in which no operators work on a regular basis except that the provisions of Section 13-413 shall apply.  [Mod 2727]

[Change Section 13-103.1.1.1 to read as follows:]

13-103.1.1.1 Commercial applications.  Completion of procedures demonstrating compliance with this code for commercial buildings shall be signed and sealed by an architect or engineer licensed to practice in the state of Florida, with the exception of buildings excluded by Section 481.229, Florida Statutes, or Section 471.003, Florida Statutes. Calculations for buildings falling within the exception of Section 471.003, Florida Statutes, may be performed by air conditioning or mechanical contractors licensed in accordance with Chapter 489, Florida Statutes, or by state of Florida certified commercial building energy raters.

Design professionals responsible under Florida law for the design of lighting, electrical, mechanical, and plumbing systems and the building shell, shall certify compliance of those building systems with the code by signing and providing their professional registration number on the energy code form provided as part of the plans and specifications to the building department.

Exception: Typed names and registration numbers may be provided in lieu of a signature where all relevant information has been included on signed and sealed plans.  [Mod 2485]