201.2  Secondary water barrier for site built single family residential structures.

A secondary water barrier shall be installed using one of the following methods when roofing replacement when reproofing.

- All joints in roof sheathing or decking shall be covered with a minimum 4 in. wide strip of self-adhering polymer modified bitumen tape applied directly to the sheathing or decking.  The deck and self adhering polymer modified bitumen tape shall be covered with one of the underlayment systems approved for the particular roof covering to be applied to the roof.

- The entire roof deck shall be covered with an approved self-adhering polymer modified bitumen cap sheet.  No additional underlayment shall be required on top of this cap sheet for new installations.

EXCEPTIONS:

1. In lieu of 1 and 2 above, buildings and structures located in HVHZ with nailable decks shall be installed in accordance with section 1518,  and  1519, which shall be deemed to meet the requirements of  a secondary water barrier

1   A asphalt impregnated 30# felt underlayment installed with nails and tin-tabs as required for the HVHZ and covered with either an approved self-adhering polymer modified bitumen cap sheet or an approved cap sheet applied using an approved hot-mop application shall be deemed to meet the requirements for the secondary water barrier.

2. A roof deck that is sufficiently fastened as prescribed in Section 201.1 and provided with a secondary water barrier as outlined above shall be deemed to meet the requirements of this section.